NYISO Power Grid Simulation Pipeline

Technical Documentation
PGScen • Vatic • NYISOLoader • System Integration

Princeton University – Operations Research & Financial Engineering
April 2026

Authors: Valentin Rudon
Advisors: Warren B. Powell

# Table of Contents

Part I – PGScen: Probabilistic Scenario Generation

1.1  Overview and Motivation

1.2  Mathematical Foundation: The GEMINI Model

1.3  Software Architecture

1.4  Data Pipeline

1.5  NYISO Adaptation

1.6  HRRR Real-Time Weather Pipeline (PGScen-2nd)

1.7  Validation and Calibration

1.8  Configuration and Hyperparameters

Part II – Vatic: Power Grid Simulation Engine

2.1  Overview and Purpose

2.2  Unit Commitment and Economic Dispatch

2.3  Software Architecture

2.4  Optimization Formulations

2.5  Data Structures and Model Representation

2.6  Simulation Workflow

2.7  Output and Results

2.8  Supported Grid Datasets

Part III – NYISOLoader: Grid Model Adapter

3.1  Overview and Design Rationale

3.2  The NYgrid Baseline (NPCC-140)

3.3  Data Mapping: NYgrid to Vatic

3.4  Generator Modeling

3.5  Transmission and Interfaces

3.6  Timeseries Integration

3.7  Fleet Updates Across Years

3.8  Supporting Modules

Part IV – System Integration

4.1  End-to-End Pipeline Overview

4.2  Data Flow Architecture

4.3  Experiment Orchestration

4.4  Stochastic vs. Deterministic Analysis

4.5  Validated Results

4.6  Known Limitations and Future Work

# Part I – PGScen: Probabilistic Scenario Generation

## 1.1  Overview and Motivation

PGScen (Power Grid Scenario Generation) is a Monte Carlo scenario generation framework developed at Princeton University for creating realistic, spatially and temporally correlated power system scenarios. The framework generates probabilistic scenarios for load demand, wind generation, and solar generation that capture the full joint distribution of forecast errors across multiple assets and time horizons.

The core motivation is that power system operations — particularly unit commitment and economic dispatch — must account for uncertainty in renewable generation and load demand. Point forecasts alone are insufficient; operators need an ensemble of plausible futures that faithfully represent the spread, correlations, and tail risks of forecast errors. PGScen provides this capability through a statistically rigorous approach based on the GEMINI (Generalized Graphical Models with Kronecker-structured covariances) framework.

The project has evolved through two major versions:

PGScen-main: The original framework, adapted from ERCOT (Texas) datasets to NYISO using real NYISO load data and synthetic NREL wind/solar data.

PGScen-2nd: An enhanced version incorporating HRRR (High-Resolution Rapid Refresh) real-time meteorological data for wind and solar generation at actual NYISO plant locations.

Key insight: PGScen scenarios capture both the spread of possible outcomes and their correlations across space (different plants) and time (hour-to-hour) — not just point forecasts with additive random noise.

## 1.2  Mathematical Foundation: The GEMINI Model

### 1.2.1  Problem Formulation

Let Y(a,h) denote the actual power output (or demand) for asset a at forecast horizon h, and let F(a,h) denote the corresponding day-ahead forecast. The deviation (forecast error) is:

D(a,h) = Y(a,h) − F(a,h)

The goal is to model the joint distribution of the deviation vector D = [D(a₁,h₁), D(a₁,h₂), ..., D(a_m,h_f)] across m assets and f = 24 forecast horizons, then sample from this distribution to generate scenarios.

### 1.2.2  Gaussianization

Raw deviations are typically non-Gaussian with heavy tails, bounded support, and possibly multi-modal structure. PGScen transforms each marginal distribution to standard Gaussian using a two-step copula transform:

Step 1 – CDF Transform: For each (asset, horizon) pair, fit either a Generalized Pareto Distribution (GPD, for load with heavy tails) or an Empirical CDF (ECDF, for wind/solar with bounded support) to the historical deviation samples. Apply the fitted CDF:

u = F_GPD(d)  or  u = F_ECDF(d)    →  u ∈ [0, 1]

Step 2 – Inverse Normal Transform: Apply the inverse standard normal CDF:

z = Φ⁻¹(u)    →  z ∈ ℝ,  approximately N(0,1)

After Gaussianization, z-scores are standardized (mean removal and unit-variance scaling) to produce approximately i.i.d. N(0,1) marginals. The GPD fitting uses R's Rsafd package via rpy2, fitting separate GPD models to the upper and lower tails with ECDF for the central region.

### 1.2.3  Separable Covariance via GEMINI

The key innovation in PGScen is fitting a separable (Kronecker-structured) covariance to the Gaussianized deviations. If we reshape the m×f deviation vector for each historical day into an f×m matrix X, the GEMINI model assumes:

Σ = Σ_spatial ⊗ Σ_temporal

where:

Σ_spatial (m × m) captures cross-asset correlations (e.g., nearby wind farms experiencing similar weather)

Σ_temporal (24 × 24) captures hour-to-hour autocorrelation (e.g., persistence of forecast errors)

⊗ denotes the Kronecker product

The GEMINI algorithm estimates these two covariance matrices by solving two nested graphical LASSO problems. For each matrix, the algorithm solves an L1-penalized maximum-likelihood estimation of the precision (inverse covariance) matrix:

A_rho = argmin_A { -log det(A) + tr(G_A · A) + ρ_A ‖A‖₁ }
B_rho = argmin_B { -log det(B) + tr(G_B · B) + ρ_B ‖B‖₁ }

where G_A and G_B are empirical correlation matrices computed from the reshaped data, and ρ_A, ρ_B are regularization parameters that control sparsity. The graphical LASSO is solved using R's glasso package. Sparse entries in the precision matrix encode conditional independence: A_rho(i,j) = 0 means assets i and j are conditionally independent given all other assets.

### 1.2.4  Scenario Sampling

Given the fitted covariance structure, new scenarios are sampled by reversing the Gaussianization pipeline:

1. Compute Kronecker square root:  L = sqrtm(Σ_spatial) ⊗ sqrtm(Σ_temporal)
2. Sample standard normal:          z ~ N(0, I_{m×f})
3. Apply covariance structure:       y = L · z
4. De-standardize:                   y' = y × σ_gauss + μ_gauss
5. Inverse-Gaussianize:              u = Φ(y')  →  d = F⁻¹(u)
6. Add day-ahead forecast:           s = d + forecast
7. Clip to physical bounds:          s = clip(s, 0, capacity)

This procedure produces correlated scenario vectors that respect the observed marginal distributions, spatial correlations across assets, and temporal persistence across the 24-hour horizon. Typical runs generate 100–1000 scenarios per day.

### 1.2.5  PCA Extension for Solar

Solar generation presents a unique challenge: nighttime hours have zero production, creating a degenerate (zero-inflated) distribution that violates the Gaussianization assumptions. PGScen addresses this through PCAGeminiEngine, which:

Detects sunrise/sunset times using the astral library for each plant location and target date

Restricts analysis to daytime-only hours (typically 6–18 depending on season)

Applies PCA dimensionality reduction (default: 90% explained variance) to the daytime deviation matrix

Fits GEMINI on the PCA-projected components, avoiding ill-conditioning from zero-inflated columns

Generates scenarios in PCA space, then back-projects to original dimensions

Pads nighttime hours with zeros in the final output

## 1.3  Software Architecture

### 1.3.1  Core Modules

| Module | Primary Class | Responsibility |
| --- | --- | --- |
| engine.py | GeminiEngine | Orchestrates the full scenario generation workflow: data loading, model fitting, scenario creation, and CSV export |
| model.py | GeminiModel | Core algorithm: Gaussianization, standardization, GEMINI fitting, scenario sampling, and inverse transforms |
| pca.py | PCAGeminiEngine | Solar-specific extension with sunrise/sunset detection and PCA dimensionality reduction |
| utils/data_utils.py | (functions) | Data loading for ERCOT, NYISO bundled, and HRRR-derived datasets |
| utils/r_utils.py | (functions) | Python↔R bridge: GPD fitting, Gaussianization, graphical LASSO, GEMINI algorithm |
| scoring.py | (functions) | Quality metrics: energy scores and variograms |

### 1.3.2  GeminiEngine Class

GeminiEngine is the primary user-facing class. Its workflow:

engine = GeminiEngine(
    hist_actual_df, hist_forecast_df,     # Historical training data
    scen_start_time,                       # Target date (UTC)
    asset_type="load",                     # "load", "wind", or "solar"
    forecast_lead_time_in_hour=10          # Hours between issue and start
)

engine.fit(
    asset_rho=0.1,       # Spatial regularization penalty
    horizon_rho=0.1,     # Temporal regularization penalty
    nearest_days=50      # Seasonal window (or None for all history)
)

engine.create_scenario(nscen=1000, forecast_df=fc_future)
engine.write_to_csv(save_dir="scenarios/", actual_dfs=act_future)

### 1.3.3  GeminiModel Class

GeminiModel (model.py, 616 lines) encapsulates the statistical machinery. Key attributes after fitting:

| Attribute | Shape | Description |
| --- | --- | --- |
| gauss_df | n_days × (m×24) | Gaussianized, standardized deviations |
| gpd_dict | dict per (asset, hour) | Fitted GPD or ECDF per marginal |
| gauss_mean | m×24 | Mean for de-standardization |
| gauss_std | m×24 | Std dev for de-standardization |
| asset_cov | m × m | Spatial covariance from GEMINI |
| horizon_cov | 24 × 24 | Temporal covariance from GEMINI |
| scen_df | nscen × (m×24) | Final generated scenarios |

## 1.4  Data Pipeline

### 1.4.1  Input Data Format

Actuals (e.g., load_actual_1h_zone_YYYY_utc.csv):

Time (index),              Zone_A, Zone_B, ...
2018-01-01 00:00:00+00:00, 500,    450,    ...

Forecasts (e.g., load_day_ahead_forecast_zone_YYYY_utc.csv):

Issue_time,                 Forecast_time,         Zone_A, Zone_B, ...
2017-12-31 18:00:00+00:00, 2018-01-01 00:00:00,   480,    430,    ...
2017-12-31 18:00:00+00:00, 2018-01-01 01:00:00,   485,    435,    ...

Forecasts use a MultiIndex with (Issue_time, Forecast_time). The issue time for day-ahead forecasts is typically 18:00 UTC on day J−1 for scenario day J, giving a 6–30 hour lead time depending on the forecast hour.

### 1.4.2  Data Loading Functions

| Function | Source | Returns |
| --- | --- | --- |
| load_ny_load_data() | Bundled NYISO CSVs | (actual_df, forecast_df) |
| load_ny_wind_data() | Bundled NREL CSVs | (actual_df, forecast_df, meta_df) |
| load_ny_solar_data() | Bundled NREL CSVs | (actual_df, forecast_df, meta_df) |
| load_ny_real_wind_data(years) | HRRR-derived CSVs | (actual_df, forecast_df, meta_df) |
| load_ny_real_solar_data(years) | HRRR-derived CSVs | (actual_df, forecast_df, meta_df) |

### 1.4.3  Deviation Extraction and Train/Test Split

For a target scenario date D, the engine computes deviations over the historical training window and provides two splitting strategies:

Seasonal window (nearest_days=N): Uses only historical days within ±N calendar days of the target date. Captures seasonality but reduces sample size.

Full history (nearest_days=None): Uses all historical data before the target date. Maximizes sample size but ignores seasonal patterns.

## 1.5  NYISO Adaptation

### 1.5.1  Real NYISO Load Data

Real NYISO load data is downloaded from mis.nyiso.com using the download_nyiso_real_load.py script. The data comprises:

Actuals: 11 NYISO zones (A through K), hourly real-time load from the "pal" (Integrated Real-Time Actual Load) dataset

Forecasts: Day-ahead zonal load forecasts from the "isolf" (ISO Load Forecast) dataset

Timezone handling: NYISO publishes in Eastern time; all data is converted to UTC for consistency

Issue time convention: For scenario day J, the forecast is issued at 18:00 UTC on day J−1

A known data quality issue exists for the MHK VL (Mohawk Valley, Zone E) zone, which exhibits a persistent positive forecast bias of approximately 60–150 MW. This bias has been decreasing over time (150 MW in 2018, 60 MW in 2025) and is attributed to rapid data center growth in the zone outpacing NYISO's load forecasting model.

### 1.5.2  Synthetic NREL Wind and Solar Data

For wind and solar, the initial adaptation uses synthetic data from the NREL PERFORM dataset, bundled in the repository:

Wind: 80 synthetic sites across New York, 2019 only, with NREL-generated forecast errors

Solar: 314 synthetic sites, 2018–2019, with NREL-generated forecasts

Forecast blocks: 06:00–05:00 UTC (corresponding to midnight–midnight Eastern)

## 1.6  HRRR Real-Time Weather Pipeline (PGScen-2nd)

PGScen-2nd introduces a complete pipeline for deriving wind and solar generation data from NOAA's High-Resolution Rapid Refresh (HRRR) numerical weather prediction model. This replaces the synthetic NREL data with real meteorological observations and forecasts at actual NYISO plant locations.

### 1.6.1  Pipeline Steps

| Step | Script | Purpose | Output |
| --- | --- | --- | --- |
| 1 | 01_parse_goldbook.py | Extract plant data from 2025 NYISO Gold Book PDF | plant_master.csv |
| 2 | 02_download_eia860.py | Fetch EIA Form 860 (lat/lon, capacity) | eia860_NY.csv |
| 3 | 03_download_uswtdb.py | Fetch US Wind Turbine Database | uswtdb_NY.csv |
| 4 | 04_build_plant_metadata.py | Fuzzy-match Gold Book ↔ EIA/USWTDB | wind_meta.csv, solar_meta.csv |
| 5 | 05_build_hrrr_timeseries.py | Download HRRR GRIB2 + power conversion | Actual/forecast CSVs |
| 6 | 06_mos_bias_correction.py | Statistical post-processing (MOS) | Corrected forecast CSVs |

### 1.6.2  HRRR Data Specifics

Archive: AWS S3 bucket noaa-hrrr-bdp-pds (public, no authentication required)

Grid resolution: 3 km Lambert Conformal (1059 × 1799 grid points)

Byte-range downloads: ~2–5 MB per file instead of the full 137 MB GRIB2 using .idx sidecar index files

Actuals: HRRR F00 (analysis fields) from every hour's run

Forecasts: HRRR F18–F41 from the 06Z run of the previous day (true day-ahead)

### 1.6.3  Power Conversion Models

Wind power conversion:

Extracts UGRD and VGRD (U and V wind components) at 80 m above ground level

Computes wind speed: ws = sqrt(U² + V²)

Applies generic IEC Class III onshore power curve: cut-in = 3 m/s, rated = 11 m/s, cut-out = 25 m/s

Cubic ramp: P = nameplate × [(ws − 3) / (11 − 3)]³

Solar power conversion:

Extracts DSWRF (downward shortwave radiation flux, i.e., GHI) and TMP (2 m temperature)

Uses pvlib DISC model to decompose GHI into DNI + DHI

Computes cell temperature using SAPM (Sandia Array Performance Model)

Converts to DC power via PVWatts model, then applies ~14% AC losses

### 1.6.4  Coverage

The HRRR pipeline covers 31 wind farms and 16 solar plants across New York State. Solar coverage increases over time as new plants come online (9 active in 2023, all 16 active by 2024). Plant metadata is matched across three databases (Gold Book, EIA-860, USWTDB) using fuzzy name matching combined with county and capacity sanity checks.

## 1.7  Validation and Calibration

PGScen-2nd includes a comprehensive validation notebook that tests scenario calibration across 20 days spanning all four seasons of 2024. Key metrics:

| Metric | Wind | Solar |
| --- | --- | --- |
| Per-plant P5–P95 exceedance | 8–12% | 7–13% |
| Per-plant P1–P99 exceedance | 1–3% | 0.5–2% |
| Fleet P5–P95 exceedance | 18–25% | 12–18% |
| Fleet P1–P99 exceedance | 3–8% | 1–4% |

The fleet-level exceedance rates exceed the per-plant rates because GEMINI's Kronecker structure underestimates cross-plant correlations. When all wind farms experience low wind simultaneously (e.g., a large-scale high-pressure system), the separable model underestimates the probability of this joint event. This is a structural limitation of the Kronecker assumption, not a data issue.

Scenario quality is also assessed using energy scores (a multivariate proper scoring rule) and variograms (spatial-temporal dependency assessment), implemented in scoring.py.

## 1.8  Configuration and Hyperparameters

| Parameter | Typical Range | Effect |
| --- | --- | --- |
| asset_rho | 0.02–0.10 | Spatial regularization (higher = sparser graph, narrower correlations) |
| horizon_rho | 0.05–0.10 | Temporal regularization (higher = fewer hour-hour correlations) |
| forecast_lead_time_in_hour | 10 (load), 12 (renew.) | Hours between issue time and scenario start |
| nearest_days | 50–100 or None | Seasonal training window; None = all historical data |
| num_of_horizons | 24 | Hours in scenario day (fixed at 24) |
| nscen | 100–1000 | Number of Monte Carlo scenarios to generate |

For wind, asset_rho is typically scaled by the inter-asset distance matrix (0.05 × d_ij / d_max) to encode the prior that distant wind farms have weaker correlations. Solar uses a lower base penalty (0.02) because solar correlations are driven by cloud systems that can affect wider regions.

# Part II – Vatic: Power Grid Simulation Engine

## 2.1  Overview and Purpose

Vatic is a Python package for power grid simulation that implements alternating day-ahead unit commitment (UC) and real-time security-constrained economic dispatch (SCED). It is a lightweight adaptation of Prescient (developed by Sandia National Laboratories) built on mixed-integer linear programming (MILP) optimization via Pyomo and power system formulations from the Egret library.

Vatic's role in the pipeline is to take the probabilistic scenarios generated by PGScen and simulate actual grid operations: which generators are turned on/off (commitment), how much power each produces (dispatch), and the resulting costs, reserves, and locational marginal prices (LMPs).

Vatic supports multiple grid datasets: RTS-GMLC (Reliability Test System), Texas-7k (7,000-bus Texas model for 2020 and 2030), and NYISO. The NYISO dataset is loaded through the NYISOLoader adapter described in Part III.

## 2.2  Unit Commitment and Economic Dispatch

### 2.2.1  Day-Ahead Unit Commitment (RUC)

The Reliability Unit Commitment (RUC) is a day-ahead planning problem that determines which generators should be committed (turned on) for the next operating day. It is typically solved once per day, at 16:00 the day before, with a 48-hour planning horizon.

The RUC optimization minimizes total expected costs subject to:

Power balance: total generation must meet forecasted demand at each bus

Generator limits: each unit must operate between Pmin and Pmax when on

Ramp rate constraints: power output cannot change faster than the unit's ramp rate (MW/hour)

Minimum up/down time: once started, a generator must run for at least its minimum up time; once shut down, it must remain off for its minimum down time

Startup costs: units incur cold/warm/hot startup costs depending on how long they have been offline

Reserve requirements: a spinning reserve margin (typically 5% of demand) must be maintained

Transmission constraints: power flows must not exceed line thermal limits (enforced via PTDF)

### 2.2.2  Real-Time Economic Dispatch (SCED)

The Security-Constrained Economic Dispatch runs every hour (or sub-hourly) with a 4-hour look-ahead horizon. It takes the commitment decisions from the RUC as fixed and optimizes the dispatch (power output) of committed generators to meet actual (realized) demand at minimum cost.

The SCED uses actual renewable generation and load demand rather than forecasts. When actual conditions differ significantly from the day-ahead forecast (e.g., a wind ramp event), the SCED must re-optimize dispatch within the constraints set by the commitment decisions. If committed capacity is insufficient, load shedding occurs at a high penalty cost ($10,000/MWh default).

## 2.3  Software Architecture

| Class | File | Lines | Purpose |
| --- | --- | --- | --- |
| Simulator | engines.py | 641 | Main simulation engine orchestrating RUCs and SCEDs |
| UCModel | models/_interface.py | 414 | Pyomo model wrapper for UC formulations |
| VaticModelData | model_data.py | 640 | Grid state: buses, generators, branches, loads, storage |
| VaticSimulationState | simulation_state.py | 377 | Tracks commitments, initial conditions, forecasts/actuals |
| PickleProvider | data_providers.py | 941 | Loads datasets, generates RUC/SCED model instances |
| VaticTimeManager | time_manager.py | 165 | Manages simulation time steps and RUC scheduling |
| VaticPTDFManager | ptdf_manager.py | 172 | Lazy PTDF constraints for transmission security |
| StatsManager | stats_manager.py | 726 | Collects results and generates output files/plots |
| GridLoader | data/loaders.py | ~500 | Abstract base class; subclasses parse grid datasets |

## 2.4  Optimization Formulations

Vatic uses formulation "modules" from the Egret library, assembled into complete UC and SCED models. Each formulation module defines a set of variables, constraints, or objective terms.

### 2.4.1  RUC Formulation Stack

| Module | Formulation | Description |
| --- | --- | --- |
| Status variables | garver_3bin_vars | Three binary variables per unit-hour: on, startup, shutdown |
| Power variables | garver_power_vars | Continuous power output bounded by commitment |
| Reserve | garver_power_avail_vars | Spinning reserve availability |
| Generation limits | pan_guan_gentile_KOW | Tight bounds on power output intervals |
| Ramping | damcikurt_ramping | Ramp-up/down limits between hours |
| Production costs | KOW_production_costs_tightened | Piecewise-linear cost curves |
| Up/down time | rajan_takriti_UT_DT | Minimum up-time and down-time constraints |
| Startup costs | KOW_startup_costs | Multi-stage (cold/warm/hot) startup costs |
| Network | ptdf_power_flow | DC power flow via Power Transfer Distribution Factors |

### 2.4.2  SCED Formulation Stack

The SCED uses simplified formulations for faster solving:

MLR_reserve_vars: Simplified reserve model for real-time

MLR_generation_limits: Simplified generation bounds

CA_production_costs: Simplified cost model

MLR_startup_costs: Simplified startup costs

The SCED retains the same network (ptdf_power_flow) and ramping (damcikurt_ramping) formulations as the RUC.

### 2.4.3  Objective Function

The total cost minimized by both RUC and SCED comprises:

No-load costs: fixed cost of keeping a unit running at minimum output

Production costs: piecewise-linear fuel cost as a function of MW output

Startup costs: cold/warm/hot startup costs when committing a unit

Shutdown costs: cost of de-committing a unit

Load mismatch penalties: $10,000/MWh for unserved load (load shedding)

Reserve shortfall penalties: $1,000/MWh for insufficient spinning reserve

Transmission violation costs: penalties for line overloads

Storage cycling costs: optional degradation cost for battery dispatch

### 2.4.4  Lazy PTDF Management

Rather than enforcing all transmission constraints simultaneously (which would create thousands of constraints), Vatic uses a lazy constraint approach via VaticPTDFManager. At each SCED solve:

The manager checks current power injections against all line limits

Lines that are congested (flow > limit) are added as active constraints

Lines that have been inactive for 5 consecutive cycles are removed

This dramatically reduces problem size while ensuring feasibility

## 2.5  Data Structures and Model Representation

Vatic represents the grid through VaticModelData, a nested dictionary structure compatible with Egret's model formulations. The top-level structure contains:

system: global parameters (baseMVA, reserve requirements, penalty costs, time keys)

elements/bus: bus names, base voltages, loads

elements/generator: fuel type, Pmin/Pmax, ramp rates, cost curves, commitment states, initial conditions

elements/branch: transmission lines with impedance and thermal limits

elements/load: time-varying load at each bus

elements/storage: battery parameters (capacity, charge/discharge rates, efficiency)

## 2.6  Simulation Workflow

The Simulator.simulate() method executes the following loop:

def simulate(self):
    # 1. INITIALIZATION: Solve RUC for the first operating day
    self.initialize_oracle()

    # 2. MAIN LOOP: iterate through all time steps
    for time_step in self._time_manager.time_steps():

        # 2a. PLANNING: solve day-ahead RUC (once per day at 16:00)
        if time_step.is_planning_time:
            self.call_planning_oracle()  # 48-hour RUC

        # 2b. OPERATION: solve real-time SCED (every hour)
        self.call_oracle()  # 4-hour SCED

        # 2c. UPDATE: advance state with SCED results
        self._simulation_state.apply_sced(current_sced)

    # 3. OUTPUT: save results
    return self._stats_manager.save_output(sim_time)

A typical 1-day simulation solves 1 RUC (the most expensive problem, taking ~10–60 seconds) and 24 SCEDs (each taking ~1–5 seconds), for a total wall-clock time of approximately 45–90 seconds per simulation day on the Kron-reduced NYISO network.

## 2.7  Output and Results

Vatic produces structured output at three detail levels:

| Detail Level | Contents |
| --- | --- |
| 0 (minimal) | Hourly system-wide: total load, generation, costs, reserves |
| 1 (default) | Level 0 + per-generator: dispatch (MW), headroom, on/off status |
| 2 (full) | Level 1 + per-bus load mismatches, per-line congestion, interface flows |

When LMP calculation is enabled (--lmps flag), Vatic re-solves each SCED with relaxed binary variables to compute locational marginal prices at each bus. LMPs decompose into energy, congestion, and loss components.

## 2.8  Supported Grid Datasets

| Dataset | Buses | Generators | Branches | Source |
| --- | --- | --- | --- | --- |
| RTS-GMLC | 73 | 158 | 120 | Reliability Test System (IEEE/NREL) |
| Texas-7k (2020) | ~7,000 | ~4,000 | ~8,800 | Texas Interconnection model |
| Texas-7k (2030) | ~7,000 | ~5,000 | ~8,800 | Texas 2030 with 50% renewables |
| NYISO | 46 (reduced) | ~243 | ~226 | Cornell NYgrid via NYISOLoader |

# Part III – NYISOLoader: Grid Model Adapter

## 3.1  Overview and Design Rationale

NYISOLoader is a GridLoader subclass that transforms the Cornell NYgrid NPCC-140 baseline model — a detailed 140-bus MATPOWER representation of the New York ISO power system — into Vatic's standardized template format for power grid simulation and optimization.

The design follows two software patterns. The Template Method pattern: GridLoader defines abstract methods that NYISOLoader implements differently from RtsLoader or T7kLoader. The Adapter pattern: NYISOLoader transforms NYgrid's JSON format into Vatic's expected dictionary structure, handling format mismatches in cost curves, generator types, and network topology.

NYISOLoader is the critical bridge between PGScen's probabilistic scenarios and Vatic's deterministic grid simulation. It maps abstract scenario data (MW by site or zone) to concrete grid elements (generators at specific buses with specific operating constraints).

## 3.2  The NYgrid Baseline (NPCC-140)

The NYgrid baseline model, developed by Liu et al. (2023) at Cornell University, provides a detailed representation of the New York power system within the larger NPCC (Northeast Power Coordinating Council) interconnection.

| Component | Count | Details |
| --- | --- | --- |
| Buses | 140 total (46 in NY) | 11 NYISO zones (A–K), rest external (PJM, NE, HQ, IESO) |
| Thermal generators | 227 | ~27,064 MW: natural gas, oil, coal |
| Nuclear generators | 6 | ~5,430 MW: Indian Point (retired 2021), Nine Mile Point, etc. |
| Dispatchable hydro | 2 major | Niagara (2,460 MW), St. Lawrence (856 MW) |
| Run-of-river hydro | 7–10 | ~879 MW at ~50% capacity factor |
| Transmission lines | 226 | Including 7 major NYISO interfaces |
| External equivalents | 4 | PJM, Hydro-Québec, New England, IESO imports |

The baseline JSON file (nygrid_baseline.json) is stored in the third_party/NYgrid/ directory and includes bus data (ID, name, zone, load, voltage, coordinates), generator data (name, bus, fuel, capacity, cost curves, UC parameters), and branch data (from/to buses, impedance, thermal limits).

### 3.2.1  Kron Reduction

For PTDF-based transmission modeling, NYISOLoader supports Kron reduction, which eliminates the 94 external buses and reduces the network to 46 NY-only buses. This dramatically improves solver performance while preserving the electrical equivalence through modified branch impedances and limits. Interface flow limits are empirically scaled by 2× to account for the approximation introduced by the reduction.

## 3.3  Data Mapping: NYgrid to Vatic

NYISOLoader performs extensive data transformation to populate Vatic's template dictionary. The major mapping tasks are described below.

### 3.3.1  Bus Mapping

NYgrid buses are filtered to the 46 New York buses (IDs 37–82) and mapped to Vatic bus names. Each bus is assigned to one of the 11 NYISO zones (A–K) using the zone lookup table:

WEST → A,  GENESE → B,  CENTRL → C,  NORTH → D,
MHK VL → E, CAPITL → F,  HUD VL → G,  MILLWD → H,
DUNWOD → I, N.Y.C. → J,  LONGIL → K

### 3.3.2  Fuel Type Mapping

| NYgrid Fuel | Vatic Code | Generator Type |
| --- | --- | --- |
| Natural Gas | NG | G (Gas) |
| Fuel Oil 2 / Fuel Oil 6 | Oil | O (Oil) |
| Coal | Coal | C (Coal) |
| Nuclear | Nuclear | N (Nuclear) |
| Wind | Wind | W (Wind) |
| Solar | Solar | S (Solar) |
| Hydro | Hydro | H (Hydro) |

## 3.4  Generator Modeling

### 3.4.1  Cost Curve Transformation

NYgrid provides cost data as linear (or optional quadratic) heat rate models in MMBTU/h. Vatic requires piecewise-linear cost curves in $/h. NYISOLoader performs the conversion:

NYgrid:   total_heat(P) = slope × P + intercept     [MMBTU/h]
Convert:  total_cost(P) = total_heat(P) × fuel_price  [$/h]
Vatic:    piecewise_linear[(P1, $1), (P2, $2), ...]

Fuel prices are zone-specific (e.g., natural gas at $3.63/MMBTU in western NY vs. $4.40/MMBTU in the Hudson Valley). The loader can use either 2019 annual averages or weekly prices from EIA data.

### 3.4.2  Unit Commitment Parameters

Startup constraints and minimum up/down times are assigned by (unit_type, fuel_type) lookup. Example entries:

| Unit Type | Fuel | Min Up (h) | Min Down (h) | Cold Start (h) | Cold Cost ($/MMBTU) |
| --- | --- | --- | --- | --- | --- |
| Steam Turbine | Natural Gas | 8 | 8 | 48 | 110 |
| Combined Cycle | Natural Gas | 4 | 4 | 12 | 70 |
| Combustion Turbine | Natural Gas | 1 | 1 | 2 | 20 |
| Steam Turbine | Coal | 24 | 24 | 168 | 150 |
| Nuclear | Nuclear | 72 | 72 | 168 | Must-run |

Three-stage startup is modeled: hot (short offline), warm (medium), and cold (long offline), each with increasing startup costs. Startup cost is computed as: cost_per_MMBTU × Pmax × heat_rate_slope.

### 3.4.3  Dispatchable Hydro

Two major hydroelectric facilities receive special treatment:

Niagara (2,460 MW, Zone A): Treaty-controlled with Pmin = 1,560 MW (~63.5% capacity factor, reflecting US–Canada treaty flow obligations) and Pmax = 2,460 MW. Modeled as must-run.

St. Lawrence (856 MW, Zone D): Pmin = 700 MW, Pmax = 856 MW. Also must-run due to international treaty obligations.

These are modeled as dispatchable generators (not renewable) because their output can be adjusted within the treaty bounds by the system operator.

### 3.4.4  Pumped Storage

| Facility | Capacity (MW) | Energy (MWh) | Bus | Zone | Charge Eff. | Discharge Eff. |
| --- | --- | --- | --- | --- | --- | --- |
| Blenheim-Gilboa | 1,160 | 9,280 | 38 | E | 87% | 92% |
| Lewiston | 240 | 2,880 | 55 | A | 85% | 90% |

Pumped storage is modeled through Egret's storage dictionary, separate from generators. Each unit has charge/discharge rates, energy capacity, round-trip efficiency, and initial state of charge (default 50%).

### 3.4.5  External Equivalents

Rather than modeling the full external networks (PJM, Hydro-Québec, New England, IESO), NYISOLoader represents them as equivalent generators at border buses:

| Interconnection | Bus | Pmax (MW) | Cost ($/MWh) | Zone |
| --- | --- | --- | --- | --- |
| PJM | RAMAPO (75) | 2,650 | 30 | G |
| Hydro-Québec | Chateauguay (48) | 1,690 | 5 | D |
| New England | Cross Sound (37) | 300 | 35 | F |
| IESO (Ontario) | Niagara (54) | 1,290 | 15 | A |

Pmax values are set at the 95th percentile of 2019 actual flows. Costs approximate the average LMP of the neighboring region. These equivalents are one-directional (imports only in the base model); export capability can be added by setting negative Pmin.

## 3.5  Transmission and Interfaces

NYISOLoader models 226 transmission lines with thermal limits and impedances from the NYgrid baseline. Seven major NYISO interface constraints are enforced:

| Interface | Limit (MW) | Direction |
| --- | --- | --- |
| DYSINGER EAST | 6,300 | West → East (Zone A→B) |
| WEST CENTRAL | 4,400 | Zone B → C |
| TOTAL EAST | 5,150 | Upstate → Downstate |
| MOSES SOUTH | 2,350 | Zone D → south |
| CENTRAL EAST | 3,225 | Zone C → E/F |
| UPNY-SENY | 5,400 | Upstate NY → Southeast NY |
| UPNY-ConEd | 5,750 | Upstate → ConEd territory |

Interface constraints are defined as linear combinations of branch flows with specified orientations. When using the Kron-reduced network, interface limits are scaled by 2× to compensate for the reduced network's modified impedance structure.

## 3.6  Timeseries Integration

NYISOLoader's create_timeseries() method builds the (gen_data, load_data) DataFrames required by Vatic. It supports two data paths:

### 3.6.1  PGScen Path (Preferred)

When pgscen_dir is specified, the loader reads PGScen-generated CSV files:

Wind: Maps per-site MW values to NYISO_W_{site_id} generators

Solar: Maps per-site MW values to NYISO_S_{site_id} generators

Load: Distributes zonal load to buses using load proportion factors from the NYgrid baseline's bus-level load data

Each column in the output DataFrame has both actual and forecast sub-columns, corresponding to real-time operations and day-ahead planning respectively.

### 3.6.2  NYISO Public Data Path (Fallback)

When PGScen data is unavailable, the loader falls back to NYISO public data:

Load: Downloads from mis.nyiso.com "pal" dataset (5-minute resolution, aggregated to hourly)

Wind: System-wide wind from "rtfuelmix" dataset, distributed to individual generators using RenewableGen.csv capacity fractions

Solar: Similar distribution from fuel mix data

The public data path provides less granularity (system-level wind rather than per-plant) and no forecast error modeling, making it suitable for deterministic simulations but not for stochastic analysis.

## 3.7  Fleet Updates Across Years

The FleetUpdater class (nyiso_fleet_updater.py, 668 lines) enables updating the generator fleet for any target year. It processes:

Retirements: Removes generators that have been decommissioned (e.g., Indian Point Units 2 and 3 in 2020–2021)

Additions: Adds new generators from the NYISO interconnection queue, NYSERDA pipeline, and Gold Book forecasts

Capacity updates: Adjusts nameplate capacity for re-rated units

Battery storage: Adds grid-scale batteries from Gold Book and interconnection queue data

The updater produces year-specific baseline JSON files (e.g., nygrid_baseline_2023.json) that NYISOLoader can load directly. Gold Book fuel codes are mapped to NYgrid names: NG→Natural Gas, UR→Nuclear, WAT→Hydro, WND→Wind, SUN→Solar, BAT→Battery.

## 3.8  Supporting Modules

| Module | Lines | Purpose |
| --- | --- | --- |
| nygrid_reader.py | 141 | Parses NYgrid JSON baseline and validates structure |
| nyiso_downloader.py | 944 | Downloads NYISO public data (zonal load, LMPs, fuel mix) with caching |
| nyiso_timeseries.py | 391 | Builds timeseries from cached NYISO data or PGScen output |
| nyiso_fleet_updater.py | 668 | Updates generator fleet for target year using Gold Book data |
| nyiso_endpoints.yaml | — | URL configuration for NYISO API endpoints |

# Part IV – System Integration

## 4.1  End-to-End Pipeline Overview

The three components form a layered pipeline for stochastic power grid simulation of the New York ISO system:

| Layer | Component | Role | Input | Output |
| --- | --- | --- | --- | --- |
| 1. Scenario Generation | PGScen | Generate probabilistic wind/solar/load scenarios | Historical NYISO data + HRRR forecasts | 100–1000 correlated scenarios per day |
| 2. Grid Model | NYISOLoader | Build power grid model with scenario timeseries | NYgrid baseline + PGScen scenarios | Vatic template + (gen_data, load_data) |
| 3. Simulation | Vatic | Run unit commitment and economic dispatch | Vatic template + timeseries | Costs, dispatch, LMPs, reserves |

This architecture is modular: each layer can be used independently or replaced. For example, PGScen scenarios could feed a different simulation engine, or Vatic could use a different scenario generator.

## 4.2  Data Flow Architecture

Data flows through the pipeline via standardized file formats:

### 4.2.1  PGScen → NYISOLoader

PGScen produces CSV files with UTC-indexed, multi-column DataFrames. The file naming convention is:

pgscen_dir/
  wind/
    wind_actual_1h_site_YYYY_utc.csv       # Actuals
    wind_day_ahead_forecast_site_YYYY_utc.csv  # Forecasts
    wind_meta.csv                           # Site metadata
  solar/
    solar_actual_1h_site_YYYY_utc.csv
    solar_day_ahead_forecast_site_YYYY_utc.csv
    solar_meta.csv
  load/
    load_actual_1h_zone_YYYY_utc.csv
    load_day_ahead_forecast_zone_YYYY_utc.csv

NYISOLoader reads these files in create_timeseries() and maps them to grid-specific generator and bus identifiers.

### 4.2.2  NYISOLoader → Vatic

NYISOLoader provides Vatic with two objects:

template (dict): Static grid data — all generators, buses, branches, cost curves, constraints, and operating limits

(gen_data, load_data) (DataFrames): Time-varying data — renewable generation and load demand with actual and forecast columns

The template is created once per grid configuration. The timeseries data is created per simulation period and can be overridden with PGScen scenario data for stochastic analysis.

### 4.2.3  Experiment-Level Scenario Override

For stochastic ensemble simulations, the experiment orchestration script applies PGScen scenarios on top of the base timeseries. For each scenario:

Wind: Per-site direct mapping via site_id → NYISO_W_{site_id}

Solar: System-level ratio scaling (limited generators in NYISOLoader)

Load: Per-zone scaling using zonal baseline load proportions

## 4.3  Experiment Orchestration

The experiments/ directory contains structured experiment scripts that orchestrate the full pipeline. A typical experiment (e.g., 001_stochastic_vs_deterministic_jul2019) follows three phases:

### 4.3.1  Phase 1: Scenario Generation

# Load historical data
load_act, load_fc = load_ny_load_data()
wind_act, wind_fc, wind_meta = load_ny_real_wind_data(years=[2019])
solar_act, solar_fc, solar_meta = load_ny_solar_data()

# Generate scenarios for each day
for day_date in dates:
    wind_engine = GeminiEngine(..., forecast_lead_time_in_hour=18)
    wind_engine.fit(asset_rho * dist/dist.max(), time_rho)
    wind_engine.create_scenario(n_scenarios, wind_fc_future)
    
    load_engine = GeminiEngine(..., asset_type="load")
    load_engine.fit(asset_rho, time_rho)
    load_engine.create_scenario(n_scenarios, load_fc_future)
    
    solar_engine = PCAGeminiEngine(..., us_state="New York")
    solar_engine.fit(asset_rho, pca_comp_rho, num_of_components=0.9)
    solar_engine.create_scenario(n_scenarios, solar_fc_future)

# Output: 100 parquet files (one per scenario)

### 4.3.2  Phase 2: Deterministic Baseline

# Load grid model with PGScen timeseries
loader = NyisoLoader(
    use_reduced_network=True,
    fuel_price_date="2019-07-15",
    pgscen_dir="../../PGscen-2nd/data/NYISO_real",
    year=2019
)
gen_data, load_data = loader.create_timeseries(start, end)

# Run deterministic simulation using day-ahead forecast
sim = Simulator(template, gen_data, load_data, solver="gurobi")
results_det = sim.simulate()

### 4.3.3  Phase 3: Stochastic Ensemble

# For each scenario (parallel across cores):
for scen_idx in range(100):
    scen_df = pd.read_parquet(f"scenario_{scen_idx:03d}.parquet")
    
    # Override timeseries with scenario values
    gen_data_s, load_data_s = apply_scenario(
        gen_data, load_data, scen_df, day_date, loader
    )
    
    # Run simulation with scenario-specific timeseries
    sim = Simulator(template, gen_data_s, load_data_s, solver="gurobi")
    results[scen_idx] = sim.simulate()

This produces 100 simulation results per day, each representing a different realization of wind, solar, and load uncertainty. The ensemble can then be analyzed for expected costs, cost distributions, reliability metrics, and value of stochastic information.

## 4.4  Stochastic vs. Deterministic Analysis

The primary research question addressed by this pipeline is: what is the value of accounting for uncertainty in power system operations? The stochastic vs. deterministic comparison proceeds as follows:

Deterministic baseline: Uses the day-ahead forecast as if it were certain. The RUC commits generators based on the single forecast, and the SCED dispatches against actual conditions. Mismatches between forecast and actual cause sub-optimal commitment decisions.

Stochastic ensemble: Generates N scenarios (e.g., 100) that span the range of plausible outcomes. Each scenario represents a different realization of the joint wind/solar/load distribution. The RUC can be run with the mean or a robust commitment, while the SCED runs against each scenario's realization.

Key metrics for comparison include: total generation cost, load shedding events (frequency and magnitude), reserve shortfalls, and LMP distributions across zones. The difference in expected cost between deterministic and stochastic operations quantifies the "value of stochastic solution" (VSS).

## 4.5  Validated Results

The pipeline has been validated against 2019 NYISO actual data for a 24-hour RUC + hourly SCED simulation:

| Metric | Simulated | Actual 2019 | Ratio |
| --- | --- | --- | --- |
| System LMP (July 8) | $27.59/MWh | $23.03/MWh | 1.20× |
| Zone J (NYC) LMP | $34.25/MWh | $24.90/MWh | 1.38× |
| Zone A (West) LMP | $25.76/MWh | $18.19/MWh | 1.42× |
| Simulation time | ~45 seconds | — | — |

The 20–40% LMP overestimation is primarily attributed to: (1) the model imports from but does not export to neighboring regions, while actual NYISO exported ~656 MW on average to New England in 2019; (2) placeholder UC parameters that may commit more expensive units than necessary; and (3) the 2× interface scaling that may over-constrain some transmission corridors.

## 4.6  Known Limitations and Future Work

### 4.6.1  PGScen Limitations

Fleet-level correlations: The Kronecker structure underestimates cross-plant correlations, producing overly narrow fleet-level scenario fans

HRRR model bias: Systematic wind speed and GHI biases are only partially corrected by MOS

Generic power curves: All wind plants use the same IEC Class III curve; no plant-specific turbine data

Fixed-tilt solar: No tracking or orientation diversity modeled

Single grid point: Large wind farms don't use spatial averaging across multiple HRRR grid cells

### 4.6.2  NYISOLoader Limitations

UC parameters: Placeholder values from industry references, not validated against NYISO-specific unit data

Hydro bounds: Static monthly averages; hourly variation from river flow data (USGS) not yet integrated

Topology frozen at 2019: Missing major transmission upgrades (CHPE, Smart Path, Clean Path NY)

No NE exports: Model only imports; actual net exports to NE average ~656 MW

### 4.6.3  Vatic Limitations

Deterministic solver: Vatic solves deterministic UC/ED; true stochastic programming (e.g., two-stage stochastic UC) is not implemented — each scenario runs independently

No demand response: The model does not include demand-side flexibility

No energy market coupling: LMPs are computed post-hoc, not co-optimized with commitment

### 4.6.4  Integration Priorities

| Priority | Item | Impact | Estimated Effort |
| --- | --- | --- | --- |
| High | Validate UC parameters against EIA-860 Schedule 3 | Reduce LMP overestimation | Medium |
| High | Add NE export capability | Fix 656 MW import bias | Low |
| Medium | Integrate USGS river flow for hourly hydro bounds | Improve dispatch realism | High |
| Medium | Wire Blenheim-Gilboa storage into template | Add 1,160 MW flexibility | Low |
| Medium | Improve fleet-level correlations in PGScen | Better tail risk capture | High |
| Low | Add plant-specific wind turbine curves | Marginal accuracy gain | Medium |
| Low | Update topology for 2025+ transmission | Required for forward studies | Medium |

# Summary

This document has described the three-layer NYISO power grid simulation pipeline developed at Princeton University:

PGScen provides the probabilistic foundation by generating spatially and temporally correlated scenarios for wind, solar, and load using the GEMINI copula model. Its NYISO adaptation uses real NYISO load data and HRRR-derived meteorological data at actual plant locations, producing calibrated scenario ensembles that capture the statistical properties of forecast errors.

NYISOLoader bridges the gap between abstract scenario data and concrete grid simulation by transforming the Cornell NYgrid 140-bus baseline model into Vatic's format. It handles the intricate details of generator cost curves, unit commitment parameters, dispatchable hydro, pumped storage, external interconnections, and transmission interfaces.

Vatic provides the optimization engine, solving daily unit commitment and hourly economic dispatch problems using mixed-integer linear programming. It produces the final operational results: generator commitments, power dispatch, system costs, and locational marginal prices.

Together, these components enable a complete stochastic analysis workflow: generate hundreds of plausible futures, simulate grid operations under each, and analyze the distribution of outcomes. This capability is essential for understanding the operational impacts of renewable energy uncertainty on the New York power system and for quantifying the value of improved forecasting and stochastic optimization.