# Session Work Log: NYISO Grid Adapter for Vatic

Dates: April 16-17, 2026

Objective: Build a complete NYISO grid adapter for Vatic (power grid UC/ED simulation), based on Cornell's NYgrid model, and integrate it with PGscen-2nd timeseries data.

# 1. Files Created and Modified

## New Files Created

| File | Lines | Purpose |
| --- | --- | --- |
| vatic/data/nyiso_loader.py | 1,260 | Main NYISO grid loader class |
| vatic/data/nygrid_reader.py | 141 | NYgrid baseline JSON reader |
| vatic/tests/test_nygrid_reader.py | 143 | Unit tests for baseline reader |
| vatic/tests/test_nyiso_smoke.py | 402 | Structural and solve smoke tests |
| third_party/NYgrid/nygrid_baseline.json | 12,617 | Serialized NPCC-140 baseline |
| third_party/NYgrid/nygrid_kron_reduced.json | 637 | Kron-reduced branch data |
| third_party/NYgrid/nygrid_branch_limits.json | 30 | Per-branch thermal limits |
| third_party/NYgrid/LICENSE | 21 | MIT license for NYgrid |
| docs/NYISO_INTEGRATION_STATUS.md | 223 | Living gap and status document |

## Modified Existing Files

| File | Changes |
| --- | --- |
| vatic/data/loaders.py | Added grid="NYISO" route to load_input() dispatcher |
| vatic/models/params.py | Added NondispatchableMarginalCost, PriceResponsiveLoad params (Egret 0.6.1 compat) |
| vatic/data_providers.py | Added price_responsive_load to model dict, _build_interfaces(), _build_storage() |

## Dependencies Installed

| Package | Version | Purpose |
| --- | --- | --- |
| scipy | 1.17.1 | Loading .mat files |
| pandas | 3.0.2 | Data manipulation |
| pyomo | 6.10.0 | Optimization modeling |
| gridx-egret | 0.6.1.dev0 | UC/ED formulations |
| CBC solver | 2.10.13 | MILP solver (Homebrew) |

# 2. NYgrid Clone and Inspection

Cloned https://github.com/AndersonEnergyLab-Cornell/NYgrid into third_party/NYgrid/. Found: npcc.mat contains the NPCC-140 bus MATPOWER case (140 buses, 227 branches, 48 generators). genParamAll.csv has 227 thermal generators with heat rate coefficients. The 48 generators in npcc.mat are just the NPCC base case; the real model replaces them at runtime with 227 thermal + 6 nuclear + 12 hydro from CSV files. Key data: bus_ny_type_zone_new.csv maps NY buses 37-82 to zones A-K. RenewableGen.csv has nuclear, hydro, wind capacity by bus. updateOpCond.m runs network reduction (MPReduction) before OPF.

# 3. Static Grid Extraction (no MATLAB)

Built Python script using scipy.io.loadmat to extract NPCC-140 baseline. Applied all modifyMPC() modifications in Python: bus type changes, 3 branch deletions, 2 E-G branch additions, interface limit updates. Merged with zone assignments, loaded 227 thermal generators from genParamAll.csv + gen_bus_assignment.csv, added 6 nuclear and 10 hydro from RenewableGen.csv. Result: nygrid_baseline.json with 140 buses, 226 branches, 243 generators, 36,764 MW total.

# 4. Reader Module and Tests

Created nygrid_reader.py with load_nygrid_baseline() and inspect_nygrid(). Created 17 unit tests verifying: all 140 buses with valid zones, generators have Pmax >= Pmin >= 0, 226 branches connect existing buses, total capacity in 37-40 GW range, Niagara/St. Lawrence/Blenheim-Gilboa specifically present.

# 5. NyisoLoader Class

Created nyiso_loader.py with NyisoLoader(GridLoader). Bypasses parent __init__ because NYgrid data format differs from CSV-based RTS-GMLC/Texas-7k. Registered in load_input() dispatcher. Template construction from JSON baseline.

# 6. Cost Curve Fix (MMBTU to $/MWh)

Verification found cost curves were in heat input (MMBTU/h), not dollars. Gas showed $8.67/MWh marginal -- a heat rate, not a cost. Fix: multiply by zone-specific 2019 fuel prices from fuelPriceWeekly_2019.csv. After: gas $33/MWh, coal $23/MWh, oil $83/MWh, nuclear $7/MWh.

# 7. Hydro Reclassification

Niagara (2460 MW) and St. Lawrence (856 MW) moved to ThermalGenerators with MustRun. Treaty-based Pmin: Niagara 1560 MW (min CF 0.635), St. Lawrence 700 MW (min CF 0.82). Blenheim-Gilboa excluded (75.8 MW in data is wrong; real plant is 1160 MW pumped storage). Vatic has full Egret storage support -- just unused by existing loaders.

# 8. UC Parameter Population

Added realistic min up/down times and 3-tier startup costs by unit type. Steam: MinUp 8-12h, startup $110-150/MW cold. Combined cycle: MinUp 4h, startup $70/MW cold. Combustion turbine: MinUp 1h, startup $20/MW cold. Nuclear: MinUp 24h, startup $500/MW.

# 9. First UC Solve and Bug Fixes

Installed Pyomo 6.10.0, Egret 0.6.1, CBC 2.10.13. Bug 1: Negative MinimumProductionCost from regression artifacts -- clamped to 0. Bug 2: Cost curve starting above Pmin for Pmin=0 generators -- aligned. Bug 3: Missing NondispatchableMarginalCost param (Egret 0.6.1) -- added. Bug 4: Missing PriceResponsiveLoad params (Egret 0.6.1) -- added. Copper-sheet solve succeeded: 24h RUC + 24 SCED, zero load shedding.

# 10. LMP Validation Discovery

Key finding: Vatic Price column = total_cost/demand (average cost $12.50), NOT marginal LMP. With run_lmps=True, Egret dual variables give $27.59/MWh avg vs actual NYISO $23.03. Range $18-30 vs actual $19-29. 20% overshoot from no NE exports.

# 11. Kron Network Reduction

PTDF solve infeasible with 140-bus network (external buses had load but no generation). Implemented standard Kron reduction: Y_red = Y_II - Y_IE * inv(Y_EE) * Y_EI. Result: 46 buses, 74 branches (67 original + 7 equivalent). Added use_reduced_network=True parameter (default).

# 12. External Import Generators

Added 4 import equivalent generators at boundary buses: PJM (bus 75, $30/MWh, 2650 MW), HQ (bus 48, $5/MWh, 1690 MW), NE (bus 37, $35/MWh, 300 MW), IESO (bus 54, $15/MWh, 1290 MW). Pmax from 2019 p95 actual flows.

# 13. Multi-segment Cost Curves

75 generators with convex quadratic heat rates get 5-point piecewise curves. Generators with concave (a2 < 0) or low-R2 quadratics fall back to linear. Convexity enforced with perturbation. Week-specific fuel prices via new fuel_price_date parameter.

# 14. Interface Constraints

Mapped 7 NYISO interfaces to 30 cross-zone branches with signed orientations. Added interface elements via Egret interface framework. Fixed key mismatch: lower_limit/upper_limit vs minimum_limit/maximum_limit. Per-branch limits failed (PTDF distribution mismatch). Aggregate interface constraints work correctly. Scaled 2x for Kron PTDF factor compensation.

# 15. Pumped Storage

Added BG (1160 MW, bus 38 GILBOA zone E) and Lewiston (240 MW, bus 55 NIAGARA zone A). Fixed BG location: RenewableGen.csv had bus 77 (wrong); real plant at bus 38. Storage reduces system LMP by ~$0.50/MWh via arbitrage.

# 16. PGscen-2nd Timeseries Integration

Implemented create_timeseries() reading from PGscen-2nd NYISO_real data. Wind: 23 sites (1,979 MW) matched by site_id. Solar: 2 sites for 2019. Load: 11 zones distributed to buses via RenewableGen.csv proportions. Fixed code ordering: PGscen generators registered before template construction.

# 17. Final Validation Results

System-level validation against July 8, 2019 (low-congestion summer weekday):

| Configuration | Avg LMP | vs Actual $22.07 |
| --- | --- | --- |
| Copper sheet (10 GW dummy) | $4.30 | 20% |
| Relaxed lines (actual load) | $16.70 | 73% (avg cost) |
| Relaxed + LMP duals | $27.59 | 120% |
| PTDF + interfaces (2x) | $28.83 | 131% |
| PTDF + interfaces + storage | $28.31 | 128% |
| Actual NYISO | $22.07 | 100% |

Complete feature set: 46-bus Kron-reduced network, 7 NYISO interface constraints, 235 thermal generators, 31 wind + 16 solar + 7 hydro NondispatchableGenerators, 2 pumped storage (BG 1160 MW + Lewiston 240 MW), 4 import generators, quadratic cost curves, UC parameters by unit type, week-specific fuel prices. 39 structural tests passing, full 24h RUC+SCED solve feasible.